Microprocessor Systems Design

EEE42101

Experiment 4: Analog Digital Conversion ADC

# Objectives:

- Get experience with analog digital conversion peripheral.

- Apply analog processing in thermistors.

- Measure temperature.

# Tools:

- PC

- Arduino Nano board

- Testing board

- MiniB-USB cable

# Components:

- NTC thermistor

- 10kΩ resistor.

Note: all material and sources of this course will be available on:

https://github.com/ashrafmalraheem/Microprocessor_Course

Feel free to download, study and modify for your own projects.

# Analog to digital converter:

Introduction:

In the previous experiments, we have been dealing with digital signals. However, real world applications don’t have digital signals only. Physical systems can generate values that have infinite value range. Therefore, it is necessary for a microcontroller or any embedded system to be able to convert physical values into digital form. That job is performed by analog to digital conversion module.

The ADC reads analog values in range of 0 – Vcc. Any signal out of this range will cause the ADC to malfunction. The converted values in between (0)b – (2n-1)b where n is the number of digits.

## Analog to digital conversion:

In order to perform analog to digital conversion, there are three steps:

- Sampling (sample and hold): take a sample of the analog input signal, because it is continuously varying.

- Quantization: Convert the sampled signal into digital representation. Usually this is done by comparator, counter and a DAC which compare the result of DAC to the input sample.

Figure 1 ADC steps: sampling and quantization

- Conversion result: convert the quantized value into its analog value. The ADC will return a value between 0b – (2n-1)b. To obtain the actual reading it needs to be converted to corresponding voltage:

| analog value=ADC reading2n*Vcc | (1) |
| --- | --- |

Figure 2 ADC block diagram ATmega328P

# Practice 1: Configure the ADC and get data.

To configure the ADC, there are many registers should be configured in order to work correctly. These steps are guided:

- Enable the ADC using ADEN bit in ADCSRA register.

- Set the prescaler so that the ADC clock is between 50-200Khz. Note that the CPU clock speed is 16MHz.

Figure 3 ADC Control and Status Register A

Figure 4 Prescaler Selection

- Select the reference voltage to be AVCC in ADMUX register.

Figure 5 ADC Multiplexer Selection Register

Figure 6 Analog Multiplexer selection bits

Note: in your Arduino nano board you should connect on of the analog inputs either to Vcc or GND using one of the resistors you have. This should give you the readings either 1023 or 0.

The previous steps should be performed once to setup the ADC. The following is to read from the ADC:

- Select the ADC input channel where the analog signal is connected to. This is done by configuring the ADMUX register bits MUX3-0 as in Figure 6 above.

- Start the conversion by setting the ADSC bit in ADCSRA register.

- Read the results from ADC register.

- Convert the result by equation (1).

Print the reading using the logging function.

# Practice 2: Measure the temperature.

To measure the temperature, one of the most common methods used in sensors is the Negative Coefficient Temperature Resistor (NTC thermistor). This resistor from its name exhibits resistor decline with temperature increase. NTC has a non-linear relationship between the resistors and the temperature. Steinhart-Hart formula demonstrates the relationship between temperature and resistor:

| T=11T0+lnRR0B | (2) |
| --- | --- |

Where:

T≡temperature in Kelvin
R≡resistor 
T0≡reference temperature=25 ℃=298.20K
R0≡reference resistor at T0
B≡temperature material constant in Kelvin.

You are going to use MF52A2103J3470 NTC thermistor, has the following characteristics:

R0= 10KΩ at T0
B=3470K

To measure the resistor, you should connect the NTC thermistor with another resistor that has very small drift/sensitivity to temperature. You can construct either of the following circuits:

Figure 7 NTC measurement circuit

Solve the potential divider equation. Then solve NTC equation and obtain the temperature reading using logging function.

Hint: You need to use ln() function in your code. Ln function in math.h is Log()

https://microcontrollerslab.com/analog-to-digital-adc-converter-working/

ATmega328P datasheet.

https://product.tdk.com/info/en/products/protection/temperature/chip-ntc-thermistor/technote/apn-chip-ntc-thermistor.html